	FIRE BEHAVIOUR FORECAST 


	FIRE No: Tulip Fire
	PREDICTION FOR: June 12th, 2025               


	WEATHER SUMMARY:   
Warm and much drier conditions will move into the fire, as an upper ridge becomes established over the NWT.   Temperatures will be near 20 C mark and RH reading look to drop below 30%.
.

Max Temp 20C Min RH 28%

Winds SE 5G20


	FIRE BEHAVIOUR FORECAST:  Today will be a drying day.   Higher temps and low humidities will begin the drying process.  This dry warmer air will dry enough that later in the afternoon flame could appear on the surface. 

	INDICIES
FFMC 56  DMC 27  DC 229   ISI 0.4  BUI 42  FWI 0.6
FIRE BEHAVIOUR SPECIFIC                                                       valid for 13:00 June 12th, 2025
Location

Fuel type

Rate of Spread (m/min)

Intensity Class
Fire Type

SS-009
C2
0.1
1
Surface
SS-009
C1
0
1
Surface


	SAFETY:  ALL CREWS TO REPORT ANY WIND SHIFTS AND SMOKE COLUMN DEVELOPMENT!!!
Ensure that all fire line personnel establish Lookouts, Communications, Escape routes (at least two) and have adequate Safety Zones (LACES). 
Report rain amounts to Operations.   
Watchout for increasing fire behaviour later in the afternoon.



	Prepared: 
	June 11th, 2025                                                                                                     

	Issued
	Jason Currie, FBAN


Page 1 of 2

