Spot Forecast

Spot forecast for: SS009-25
Issued by True North Weather at 1:30 pm Wednesday June 11, 2025. Forecast valid for Today, Tonight and Thursday with an outlook for the next three days.

Synopsis

A very weak weather pattern will reside over the NWT and the fire site today and Thursday. The wind regime will be very light both days. This forecast indicates a preferred east-southeast direction today and Thursday. However, the directions could well be quite variable, and may be greatly influenced by the fire itself. 
Temperatures will be trending higher, but still quite cool today. Some lingering cloud could provide the chance of a brief shower today, but any amount will be very meager. RH reading will remain quite elevated today. 
Warmer and much drier conditions will move in on Thursday, as an upper ridge becomes established over the NWT. Temperatures will be near the 20C mark. And RH readings look like they could drop just below 30%.  

Forecast

TODAY: VARIABLE CLOUD. CHANCE BRIEF SHOWER  
	MAX TEMP
	15

	MIN RH
	45

	PRECIP
	Chnc .0.5mm

	1 pm Values
	13  60%  ELY 5G10-20

	3 pm Values
	14  50%  ELY 5G10-20

	6 pm Values
	15  45%  SE 5G10-15


TONIGHT: CLEARING TO A FEW CLOUDS 
	MIN TEMP
	6

	RH RECOVERY
	95% GOOD

	PRECIP
	NIL

	8pm Values
	14  50%  SE 5G10

	10 pm Values
	12  60%  SE 5G10


THURSDAY:  MAINLY SUNNY. WARMER AND DRIER
	MAX TEMP
	20

	MIN RH
	28%

	PRECIP
	NIL

	1 pm Values
	18  33%  SE 5G15 

	3 pm Values
	19  30%  SE 5-10G15-20

	6 pm Values
	20  28%  SE 5-10G15-20

	8 pm Values
	19  30%  SE 5-10G15

	10pm Values
	15  35%  SE 5G10


Atmospheric Stability and Inversions
A defined inversion will form overnight that will require a temperature of about 15 to fully erode. This will occur near 11:00 am Thursday. The very light wind field will make is difficult to quickly disperse and smoke build up, and good venting could be a problem. 
Confidence

Generally good. The light wind regime could well result in variable wind directions today and Thursday. The fire itself may have a greater influence on the wind field. 
Outlook for the Next 3 Days
The upper ridge will continue to build over the NWT, and actually become an Omega blocking pattern over the weekend. Temperatures will climb progressively higher into the weekend, with RH reading correspondingly dropping. By Friday RH reading should be into the upper twenties. And by Sunday RH values look to fall into the lower twenties. This will couple will warm temperatures to produce the likelihood of cross-over come Sunday. 
The wind regime will continue quite light through the outlook period, which could make for difficult venting conditions. 

No precipitation is anticipate through the outlook period.

	Date
	Max Temp
	Min RH%
	Ave Afternoon

Wind (km/h)
	1pm

Temp
	1pm RH%
	Weather
	Precip (mm)

	FRI JUN 13
	22
	27
	NLY 5-10G15-20
	20
	28
	MAINLY SUNNY. DRY
	NIL

	SAT JUN `14
	24
	27
	ELY 10G15-20
	23
	28
	MAINLY SUNNY. DRY
	NIL

	SUN JUN 15
	24
	20-22
	VRBL 5G15-20
	22
	25
	SUNNY. VERY DRY
	NIL


END/Dupilka
Page 3 of 3

